TURNPIKE CHEMICALS LLC — LAKE CHARLES, LA

T-02-003 SHELL THICKNESS REVIEW

Governing corrosion rate and date to minimum required thickness — as at 06/15/2026

| Item | Detail |
| --- | --- |
| Asset | T-02-003 (Bund 2), derivative blend storage (Spec TC-PROD-BLD-06). 17 m (55.8 ft) dia. × 12.2 m (40 ft), five 8 ft courses, fixed cone roof, concrete ringwall. API 650 7th Ed. (1980); placed in service 09/14/1987; in-service code API 653. |
| Question | Which rate of shell loss properly applies at the governing location, and on what date does the shell there reach its minimum required thickness (t-min)? Basis currently carried: FFS TC-MECH-FFS-0059 Rev 1 and workbook TC-MECH-CALC-0075 Rev 1 (1.9 mpy; 46.3 years). |
| Assessment date | 06/15/2026 (records on file at that date; listed in Section 2) |
| Document no. / status | Number to be allocated by Engineering Administration (not self-allocated, per S. Lee memo of 12/09/2024). Rev A, draft for check by the Mechanical Integrity Engineer and review by the site API 653 inspector. This is an engineering input to the FFS re-issue. It is not itself a certified FFS assessment. |

# 1. Result

The best like-for-like evidence puts the rate of shell loss at the governing location of T-02-003 at 3.75 mpy. It was measured at the nearest matched station, 12 in above the governing point, and is if anything higher at the point itself. That is about twice the 1.9 mpy carried in the FFS and the workbook. At this rate the shell there reaches t-min (0.215 in) on 03/01/2049 (about March 2049), not around the end of 2071 as now carried. The remaining life as at 06/15/2026 is 22.7 years, not 45.5 years (46.3 years counted from 09/2025).

| Item | Value | Source / basis |
| --- | --- | --- |
| Governing location | Shell course 1, south quadrant, 6 in above the bottom-of-shell weld line (Ashgrove grid ref. C1-S-6). On Merrowfield's datum this point is 6 in below the top of the ringwall, which is below Merrowfield's lowest station. | R1 T-02-003 row 20; R3 §4.2; Sections 3 and 5 |
| Governing thickness | 0.303 in, read 09/12/2025. Used as recorded but not yet validated (Section 6). | R1 T-02-003 row 20 |
| Readings that set the rate | 0.349 in (04/2021) → 0.334 in (04/28/2025) at Merrowfield C1-S-6, 18 in above the weld. Same grid and same datum. Loss of 0.015 in over 4.0 years, the same 0.015 in in the N, E, S and W quadrants. | R2 T-02-003 rows 6, 10, 14, 18, 22 (baseline CB-UT-2021-019); R2 Summary A11 |
| Rate applied | 3.75 mpy (0.00375 in/yr). Basis: short-term, like-for-like, pre-event (04/2021 – 04/2025). | Sections 7 and 8 |
| Minimum required thickness | 0.215 in (record value; check range 0.189 – 0.223 in) | R7 §2–3; R8 §4; R9; Section 9 |
| Wall above t-min at last reading | 0.303 − 0.215 = 0.088 in (09/12/2025) | Section 8 |
| Further loss to 06/15/2026 | 276 days (0.756 yr) × 0.00375 in/yr = 0.0028 in | Section 8 |
| Wall above t-min as at 06/15/2026 | 0.0852 in (estimated thickness 0.3002 in) | Section 8 |
| Remaining life as at 06/15/2026 | 0.0852 ÷ 0.00375 = 22.7 years | Section 8 |
| Date t-min is reached | 03/01/2049 (about March 2049) | Section 8; sensitivity in Section 9 |
| Basis now carried | 1.9 mpy, long-term from construction nominal; 46.3 years from 09/12/2025 → about 12/31/2071 | R7 §2–3; R9 T-02-003 row 9 and Summary row 11 |

Key points

The 0.031 in 'step' between the 04/2025 Merrowfield and 09/2025 Ashgrove surveys at 'C1-S-6' is not metal loss. Merrowfield measures elevation from the top of the ringwall. Ashgrove measures from the bottom-of-shell weld line, which is 12 in lower. The two 'C1-S-6' readings are therefore 12 in apart on the shell, and 0.031 in is the thickness gradient over that 12 in. Read as loss, it would give 83 mpy and put t-min at 10/2026.

With the datums aligned, the two surveys agree. At the eight points both contractors read (48 and 72 in above the weld), Ashgrove reproduces Merrowfield to within 0.001 – 0.002 in. The same alignment holds on T-02-001 and T-02-002.

The 2025 interval is too short to give a rate. It covers 136 days, only 88 of them after the 06/16/2025 blend change, and the change is within reading tolerance. The only reliable like-for-like rate is the four-year 2021 → 2025 rate: 3.75 mpy at the lowest matched station, 12 in above the governing point. Measured rates increase towards the weld.

The 1.9 mpy long-term figure is not representative. It averages 38 years from an assumed nominal start thickness and is half the rate now measured.

The governing reading (0.303 in) needs re-validation. It was taken in a calibration period that Ashgrove's own procedure voids, and it has not been validly re-shot. It is still used, because the instrument error found was over-reading, so the true wall is at or below the recorded value. A calibrated re-survey is needed. Adverse sensitivities (Section 9) bring the date forward to 2044 – 2048.

# 2. Records relied on

| Ref | Record (file and location used) | Rev / date | Used for |
| --- | --- | --- | --- |
| R1 | Ashgrove NDE — UT Thickness Survey Workbook AG-UT-2025-0917 (Ashgrove_UT_Survey_2025.xlsx); sheets Cover, T-02-003, T-02-001, T-02-002, Summary, Approval | Issued 09/23/2025; T-02-003 read 09/12–09/13/2025 | Governing thickness; matched-point check; datum cross-check |
| R2 | Merrowfield Inspection Services — UT Thickness Survey Workbook MIS-RPT-2025-TC-LC-01 (Merrowfield_UT_Survey_2025.xlsx); sheets Cover, T-02-003, T-02-001, T-02-002, Summary | 04/2025 block; T-02-003 read 04/28–04/29/2025 | 2025 readings and 04/2021 baselines (CB-UT-2021-019) → rate |
| R3 | Ashgrove UT Procedure AG-UT-014 Rev 2 — §3, §4.1, §4.2, §4.4, §5.1, §6.1 | 12/06/2024 | Ashgrove datum and grid; calibration tolerance and void rule |
| R4 | Merrowfield UT Procedure MIS-UT-003 Rev 3 — §4, §5.1, §5.3, §5.4, §6.2 | 05/12/2024 | Merrowfield datum and grid; tolerance |
| R5 | Ashgrove calibration log AG-UT-F01-2025-0912 — entries 8–12 and remarks | 09/08–09/15/2025 | Out-of-tolerance check on T-02-003 course 1 |
| R6 | Ashgrove field data sheet AG-FLD-2025-0917-C1 (T-02-003 course 1) | 09/17/2025 | Datum used on T-02-003; validity of the 0.303 in reading |
| R7 | FFS assessment TC-MECH-FFS-0059 Rev 1 (FFS_Assessment_T-02-003.pdf) — §2, §3 | 11/20/2025 | Carried rate (1.9 mpy) and t-min (0.215 in) |
| R8 | External inspection report TC-MECH-INSP-0163 Rev 1 — §3, §4, §5, §6 (F-01), §7 | 01/14/2026 | 'Step' finding; t-min by course; 1.1 mpy |
| R9 | Corrosion Rate and Remaining Life Workbook TC-MECH-CALC-0075 Rev 1 — Cover A18; T-02-003 sheet; Summary row 11 | 05/22/2026 | Carried rate, t-min, remaining life (46.3 yr); site rate convention |
| R10 | Tank Inspection Program 2026 TC-MECH-PROG-0014 Rev 3 — row 19; Notes | 06/10/2026 | Programme status (Action Required Y) |
| R11 | Tank Inspection Plan TC-MECH-PLAN-0203 Rev A (draft) — §2, §3 | 06/10/2026 | Interval basis; datum section left blank |
| R12 | Process Data Sheet TC-PROC-DS-0203 Rev 2 | 06/24/2025 | SG 0.79; max. fill 34 ft; blend revision |
| R13 | Construction & Nameplate Records Bunds 1 & 2, CBC-ENG-TANK-CONS-BUND12 — §1, §7.1, §7.3 | 08/14/2019 | Diameter, height, design SG 0.78; in service 09/14/1987; MTRs not in pack |
| R14 | Tank Design Data Register TC-MECH-REG-0011 Rev 1 — T-02-003 row, note 2 | 04/22/2026 | Nominal C1 0.375 in (design-drawing value, not measured) |
| R15 | Legacy Inspection Report Index 2016–2023 — CB-UT-2018-020, CB-UT-2021-019 | Closed 09/26/2024 | UT history: 2018 summary letter only; 2021 Delta Coast survey |
| R16 | Process change note PCN-2025-039 (S. Ali e-mail 06/18/2025); MOC-2025-019 | Effective 06/16/2025 | Date of the blend change ('event') |
| R17 | FFS Assessment Guidance TC-MECH-PROC-0007 Rev 2 — §4 | 06/03/2026 | Rate basis and interval must be stated |
| R18 | MI Written Program TC-MECH-PROG-0001 Rev 1 — §3, §5.2 | 03/2025 | Certified-evidence rule; service-change reassessment |
| R19 | API certificate 653-38472, S. David | Issued 02/20/2026 | Certificate lapse 11/30/2025 – 02/20/2026 |
| R20 | Integrity Data Handover Note CB-LC-HAND-2024-01; fitter note 'T02003 access and staging' (H. Chapman, '12/9'); FFS Tracker TC-MECH-TRK-0002 | 10/01/2024; 09/12/2025; 06/12/2026 | Contractor procedures not held; course 5 access; tracker entry |

# 3. The two 2025 surveys use different elevation datums

## 3.1 What the records say

| Item | Merrowfield (R4, R2) | Ashgrove (R3, R1, R6) |
| --- | --- | --- |
| Elevation datum | Top of the foundation ringwall, where it meets the outside face of the shell (R4 §5.1) | Bottom-of-shell weld line: the outer toe of the shell-to-bottom fillet weld (R3 §4.2) |
| Relationship stated | — | "On Turnpike Chemicals' field-erected tanks at Lake Charles, this datum stands 12 in (305 mm) below the top of the foundation ringwall" (R3 §4.2) |
| Course-1 stations | 6, 18, 36 and 60 in above datum (R4 §5.3) | Default 6, 24, 48 and 72 in above datum (R3 §4.1); used as-is on T-02-003 |
| Datum shown in workbook? | No. The datum is 'defined in … Section 5 … and is not restated' (R2 Cover B23; T-02-003 note, row 47) | No. The datum is 'defined in the procedure … not restated' (R1 Cover B20), and no transposition is applied in the workbook (R3 §4.4) |
| T-02-003 field record | — | R6: 'As per AG procedure §4.2' ticked; 'Owner-specified datum' not ticked |

None of the inspection report (R8 §4 compares 'grid points carrying the same nominal elevations'), the FFS (R7) or the workbook (R9) states or applies a datum. In the draft inspection plan (R11 §3), the bullets under 'Grid density and datum convention' are blank.

## 3.2 Transposition onto one datum

Elevation above the weld line = Merrowfield elevation + 12 in. Ashgrove elevations are already measured from the weld line.

| Elevation above weld line (in) | Merrowfield label (above ringwall) | Ashgrove label on T-02-003 (above weld) | Comparable? |
| --- | --- | --- | --- |
| 6 | — (below Merrowfield grid) | C1-x-6 | No — Ashgrove only |
| 18 | C1-x-6 | — | No — Merrowfield only |
| 24 | — | C1-x-24 | Between Merrowfield 18 and 30 |
| 30 | C1-x-18 | — | No — Merrowfield only |
| 48 | C1-x-36 | C1-x-48 | Yes — same point |
| 72 | C1-x-60 | C1-x-72 | Yes — same point |

## 3.3 The readings confirm the 12 in offset

Matched points on T-02-003 (48 and 72 in above the weld). Ashgrove (09/12/2025) is lower than Merrowfield (04/28/2025) by 0.001 in in the south quadrant and 0.002 in in N, E and W, at both elevations. All eight pairs agree within 0.002 in. At the 2021–2025 rates for these elevations, 136 days of service would remove 0.0008 – 0.0011 in.

Ashgrove's 24 in station is midway between Merrowfield's stations at 18 and 30 in above the weld. In all four quadrants the Ashgrove reading is 0.0005 in below the Merrowfield midpoint, e.g. south: 0.340 against (0.334 + 0.347) ÷ 2 = 0.3405 in.

Sister tanks T-02-001 and T-02-002, read by the same two contractors in the same campaigns (R1, R2). On those tanks Ashgrove placed course-1 stations at 18, 30, 48 and 72 in above the weld, which are Merrowfield's 6, 18, 36 and 60 in transposed. It reproduced Merrowfield's readings within 0.000 – 0.004 in at every point; for example, T-02-002 south: 0.263 / 0.265 / 0.276 / 0.292 in against 0.263 / 0.266 / 0.277 / 0.293 in. On T-02-003 Ashgrove used its default 6/24/48/72 in grid instead, so only the label '6' coincides.

## 3.4 Consequence: the 0.031 in 'step' is a datum artefact

Merrowfield's C1-S-6 is 18 in above the weld. Ashgrove's C1-S-6 is 6 in above the weld. The 0.031 in step in R8 §4 and finding F-01 compares these two different points. It is exactly 0.031 in in all four quadrants (N 0.337 / 0.306, E 0.335 / 0.304, S 0.334 / 0.303, W 0.336 / 0.305 in). That is what a fixed elevation offset on a smooth thickness gradient produces. It is not what 4½ months of corrosion would produce: real loss would also show at the matched points, where the change is 0.002 in or less. No corrosion rate can be derived from the step.

Figure 1 — Course 1, south quadrant. (a) As labelled: the two 'C1-S-6' readings sit 0.031 in apart. (b) With Merrowfield elevations moved up 12 in onto the weld-line datum, the 2025 surveys lie on one profile and coincide at the matched points. The Ashgrove 6 in reading is the only measurement in the lowest 18 in of the shell. Data: R1, R2.

# 4. Course 1 thickness on a common datum

| Elevation above weld (in) | Merrowfield grid | Ashgrove grid | CB-UT-2021-019 04/2021 (in) | Merrowfield 04/28/2025 (in) | Ashgrove 09/12/2025 (in) |
| --- | --- | --- | --- | --- | --- |
| 6 | — | C1-S-6 | — | — | 0.303 |
| 18 | C1-S-6 | — | 0.349 | 0.334 | — |
| 24 | — | C1-S-24 | — | — | 0.340 |
| 30 | C1-S-18 | — | 0.360 | 0.347 | — |
| 48 | C1-S-36 | C1-S-48 | 0.369 | 0.357 | 0.356 |
| 72 | C1-S-60 | C1-S-72 | 0.377 | 0.367 | 0.366 |

Table 4 — South quadrant, course 1. The 2021 values are the baselines Merrowfield carries 'at the same grid location' (R2 Cover B25). R2 Summary A11 states that change values are computed only where the two surveys 'share both a grid reference and a datum'. All four quadrants are tabulated in Appendix A.

The Merrowfield visual note (R2 T-02-003 row 7) records 'generalized thinning on the lower part of course 1, reducing with elevation; no localized pitting evident'. The profile matches that note: the thinnest metal is nearest the shell-to-bottom weld, and loss falls off steadily with height. The 2021 readings at 72 in above the weld (0.377 – 0.380 in) are above the 0.375 in nominal, so the as-built plate was thicker than nominal.

# 5. Governing location

| Course | Lowest 2025 reading (grid) | t-min (in) | Wall above t-min (in) | Highest like-for-like rate 2021→2025 (mpy) | Indicative life (yr) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.303 (Ashgrove C1-S-6) | 0.215 | 0.088 | 3.75 | 23.5 |
| 2 | 0.293 (Ashgrove C2-E-24 / C2-W-24) | 0.170 | 0.123 | 2.25 | 54.7 |
| 3 | 0.237 (Ashgrove C3-E-24) | 0.140 | 0.097 | 1.75 | 55.4 |
| 4 | 0.241 (Ashgrove C4-E-24) | 0.140 | 0.101 | 1.00 | 101 |
| 5 | 0.243 (Ashgrove C5-E-24) | 0.140 | 0.103 | 0.75 | 137 |

Table 5 — t-min for courses 2–5 from R8 §4. R9 gives 0.171 / 0.127 in for courses 2 / 3, which does not change the ranking. Rates are from Merrowfield readings against CB-UT-2021-019 baselines (Appendix B).

Course 1 governs, as the FFS, the inspection report and the workbook also conclude. The Ashgrove summary (R1 Summary row 8) lists course 3 as 'governing', but only because it has the lowest absolute reading (0.237 in); that ignores t-min. Within course 1, the thinnest band is 6 in above the weld: 0.303 – 0.306 in at N, E, S and W, lowest at S. The governing location is therefore course 1, south, 6 in above the bottom-of-shell weld line. Two coverage gaps remain:

the 6 in band was read at only four of the eight octants (NE, SE, SW and NW were read at 24 in only), so the circumferential minimum at that height is not known;

apart from Ashgrove's 6 in readings, nothing has ever been measured in the lowest 18 in of the shell.

# 6. Reliability of the readings

## 6.1 Readings used for the rate (Cordell Bayou 04/2021 and Merrowfield 04/2025)

Merrowfield 04/28/2025. The procedure tolerance is ±0.002 in (R4 §4), and every shift check was within ±0.002 in (R2 Summary A12). Surfaces were clean bare metal; two readings were taken at 90° per location (R4 §6.2); a Level III sign-out is recorded (R2 Summary A13).

CB-UT-2021-019 (Delta Coast, 04/2021), held as the baseline column in R2. Merrowfield computes change only where grid and datum are shared (R2 Summary A11). The data behave like real wall loss rather than a datum shift: the 2021 profile is smooth, the 2021→2025 losses fall steadily with height (0.015, 0.013, 0.011–0.012, 0.009–0.010 in), and they are the same in all four quadrants.

Independent corroboration. Ashgrove's matched readings reproduce Merrowfield's 2025 values within 0.001 – 0.002 in (Section 3.3).

Limitations. The original CB-UT-2021-019 report (indexed as transferred, R15) was not among the records reviewed, and Delta Coast's procedure is not held on site (R20). The 2021 datum therefore rests on Merrowfield's like-for-like statement. Merrowfield's Level III sign-out is dated 04/28/2025, but the T-02-003 readings run to 04/29/2025; this should be clarified with Merrowfield. The day of the 2021 survey is not recorded, so 4.0 years is used; an early-April 2021 survey would give 4.07 years and 3.68 mpy, so 4.0 years is the conservative choice. Reading tolerance on a 0.015 in loss is about ±0.004 in. The identical result in four quadrants indicates low scatter.

## 6.2 Ashgrove course-1 readings of 09/12/2025: calibration

| Entry | Date / time | Event (tank / area) | 0.300 in block read | Deviation | Result |
| --- | --- | --- | --- | --- | --- |
| 8 | 09/12/2025 07:00 | Start of shift (T-02-003 course 1) | 0.299 | −0.001 | In tolerance |
| 9 | 09/12/2025 11:20 | Ambient drift > 20 °F (T-02-003 course 1) | 0.302 | +0.002 | In tolerance |
| 10 | 09/12/2025 17:45 | End of shift (T-02-003 course 1 / course 2 start) | 0.306 | +0.006 | OUT OF TOLERANCE |
| 11 | 09/13/2025 07:05 | After re-zero and transducer face dressing (courses 2–5) | 0.300 | 0.000 | In tolerance |

AG-UT-014 §3 sets the tolerance at ±0.003 in and states: 'Out-of-tolerance findings void all readings taken since the previous in-tolerance calibration; affected grid points shall be re-shot.' Every reading taken on 09/12/2025 after 11:20 is therefore void. The procedure (§6.1) requires each workbook entry to carry a calibration timestamp, probe serial and technician initials, but the T-02-003 sheet has none, so the void readings cannot be identified. No re-shoot of course 1 is logged; the 09/13 entries cover courses 2–5. The instrument was over-reading (+0.006 in), so any reading in the void window overstates the wall by up to 0.006 in. The close agreement at 24, 48 and 72 in suggests those stations were not materially affected. The 6 in stations, including the governing one, have nothing to be checked against.

## 6.3 Field data sheet AG-FLD-2025-0917-C1 (09/17/2025) is not a valid re-shoot

Instrument 's/n AG-1147' is not the campaign instrument (38DL-11427, R1 Cover B11). It does not appear in the calibration log, which covers only 38DL-11427 and has no entries after 09/15/2025 (R5).

The technician, D. Ferris, is not on the workbook cover or approval sheet (R. Alvarez, J. Boudreaux; R1).

The reviewer is given as 'M. Okafor, Ashgrove supervising technician'. M. K. Okafor is the Merrowfield readings technician named on the Merrowfield workbook cover (R2 Cover B17). The reviewer's identity needs to be confirmed.

Seven of the eight stations were read through intact coating; only C1-S was taken to bare metal ('C1-S bare metal patch, remainder coated — DF'). R3 §5.1 requires coating thicker than 0.010 in, or of unknown condition, to be removed to bright metal, and the sheet records no coating thickness.

No station elevations are recorded; stations are given only as C1-N … C1-NW.

'Readings transcribed to the issued workbook without adjustment' is untrue for 7 of the 8 stations. For example, the sheet gives C1-N 0.329 and C1-E 0.315, while the workbook gives C1-N-6 0.306 / C1-N-24 0.343 and C1-E-6 0.304 / C1-E-24 0.341. Only C1-S matches: its triplet 0.303 / 0.305 / 0.304 is identical to the workbook's C1-S-6 entry, even though the workbook records a different instrument and survey dates of 09/12–09/13.

The provenance of the 0.303 in value is therefore uncertain. It came either from the void window on 09/12 or from an uncontrolled instrument on 09/17.

## 6.4 How the 0.303 in reading is treated

The reading is not validated. It is nonetheless used as the governing thickness, for three reasons:

it is the only measurement in the lowest 18 in of the shell, since Merrowfield's grid starts 18 in above the weld;

it agrees with the other three quadrants at the same height (0.304 – 0.306 in) and lies on the profile measured above it (Figure 1b);

the error mechanism found (instrument over-reading) means the true wall is at or below 0.303 in. Replacing it with Merrowfield's 0.334 in from 12 in higher would be non-conservative (basis G, Section 7).

A worst-case allowance (0.303 − 0.006 = 0.297 in) is included in the sensitivity analysis (Section 9). The reading must be confirmed by a calibrated re-survey (Section 10, action 1).

# 7. Selecting the rate

| Ref | Basis | Readings and interval | Rate (mpy) | t-min date implied | Adopted |
| --- | --- | --- | --- | --- | --- |
| A | Long-term from construction nominal, at Ashgrove C1-S-6 (FFS R7 §2; workbook R9) | 0.375 in nominal (09/14/1987) → 0.303 in (09/12/2025); 38.0 yr | 1.9 | ≈ 12/31/2071 (46.3 yr) | No |
| B | Long-term from construction nominal, at Merrowfield C1-S-6 (R8 §5) | 0.375 → 0.334 in (04/2025); 37.6 yr | 1.1 | n/a — not governing | No |
| C | Short-term across the blend change at the 'same nominal elevation': Merrowfield C1-S-6 → Ashgrove C1-S-6 (implied by R8 §4 / F-01) | 0.334 → 0.303 in; 136 days | 83 | 10/03/2026 | No |
| D | 2021 baseline at 'C1-S-6' → Ashgrove C1-S-6 | 0.349 → 0.303 in; 4.4 yr | 10.5 | 01/25/2034 | No |
| E | Short-term after the event, at physically matched points (48 and 72 in above weld) | Merrowfield 04/2025 → Ashgrove 09/2025; change 0.001 – 0.002 in; 136 days (88 after the event) | 2.7 – 5.4 (not resolvable) | 01/2042 – 06/2058 | No |
| F | Short-term, like-for-like, at the lowest matched station (18 in above weld): CB 2021 → Merrowfield 2025 | 0.349 → 0.334 in (S); 0.015 in in all four quadrants; 4.0 yr | 3.75 | 03/01/2049 | Yes |
| G | Rate F applied to Merrowfield's 0.334 in, disregarding the Ashgrove 6 in readings | As F | 3.75 | 01/20/2057 | No |

## 7.1 Why basis F is adopted

Like for like. The two readings share a grid point, a datum and a physical location.

The interval resolves the rate. A loss of 0.015 in over 4.0 years is several times the ±0.002 in reading tolerance, and the loss is identical in all four quadrants.

It is the nearest matched station to the governing point, 12 in above it.

It is a lower bound at the governing point. Measured rates rise towards the weld (Table 7.2). A straight-line fit through the south-quadrant profile gives about 3.9 mpy at 6 in; this is carried as a sensitivity rather than adopted, because it is an extrapolation.

It is consistent with the post-event data. At the matched points, 136 days at this rate profile would remove 0.0008 – 0.0011 in; 0.001 – 0.002 in was observed. There is no sign of a step change after the 06/16/2025 blend change (R16), although the interval is too short to rule one out.

It matches the site's own convention. Workbook R9 (Cover A18) says the long-term rate is to be taken 'from the earliest held UT baseline at that grid where one does [exist]'. For T-02-003 that baseline is CB-UT-2021-019, which gives 3.75 mpy on the matched grid. The workbook instead used construction nominal.

For the record under TC-MECH-PROC-0007 §4 (R17), the basis is: short-term (pre-event), service interval 04/2021 – 04/2025, Merrowfield grid C1-x-6 (18 in above the bottom-of-shell weld).

| Elevation above weld (in) | Merrowfield grid | N (mpy) | E (mpy) | S (mpy) | W (mpy) |
| --- | --- | --- | --- | --- | --- |
| 18 | C1-x-6 | 3.75 | 3.75 | 3.75 | 3.75 |
| 30 | C1-x-18 | 3.25 | 3.25 | 3.25 | 3.25 |
| 48 | C1-x-36 | 2.75 | 2.75 | 3.00 | 2.75 |
| 72 | C1-x-60 | 2.25 | 2.25 | 2.50 | 2.25 |

Table 7.2 — 2021 → 2025 like-for-like rates on course 1: loss in inches ÷ 4.0 years (R2). The rate rises steadily towards the weld.

## 7.2 Why the other bases are not adopted

A (1.9 mpy, carried). This is a 38-year average that assumes the shell started at the 0.375 in nominal. (i) It is half the rate measured like for like over the last four years 12 in above the same point. At that point the long-term figure is 1.1 mpy against a current 3.75 mpy. Loss has accelerated, so the long-term average understates the current rate. (ii) The start thickness is not known. The 2021 readings at 72 in above the weld exceed nominal, and the 'fabricator MTR set' the FFS cites is not in the construction pack (R13 §1: MTRs 'not reproduced in this pack'). An as-built thickness above nominal makes the true long-term rate higher than 1.9 mpy. (iii) The site convention calls for the earliest held UT baseline (Section 7.1).

B (1.1 mpy). It has the same defects as A, and the location does not govern.

C (83 mpy, t-min by 10/2026). It compares two points 12 in apart (Section 3), and the matched points, which change by 0.002 in or less, contradict it.

D (10.5 mpy). It makes the same datum error, pairing a 2021 reading 18 in above the weld with a 2025 reading 6 in above it.

E (2.7 – 5.4 mpy). These points are physically matched and do span the event, but they cannot set a rate. The interval is 136 days, only 88 of them after the event. The change of 0.002 in or less is inside the combined tolerance (±0.002 in Merrowfield plus ±0.003 in Ashgrove, i.e. ±0.005 in, or about ±13 mpy over 0.37 yr). The Ashgrove readings fall in the voided calibration period. The points are not at the governing elevation. Basis E is used only as a check that no large step occurred.

G (t-min by 01/2057). Non-conservative. It ignores the only measurement in the lowest 18 in of the shell, and the calibration error found would make the true wall thinner, not thicker.

# 8. Calculation

| Step | Working | Result |
| --- | --- | --- |
| 1 Rate (basis F) | S: 0.349 − 0.334 = 0.015 in. N: 0.352 − 0.337; E: 0.350 − 0.335; W: 0.351 − 0.336 — all 0.015 in. Interval 04/2021 → 04/28/2025 = 4.0 yr. 0.015 in ÷ 4.0 yr = 0.00375 in/yr | 3.75 mpy |
| 2 Wall above t-min at last reading | 0.303 in (Ashgrove C1-S-6, 09/12/2025) − 0.215 in | 0.088 in |
| 3 Bring forward to 06/15/2026 | 09/12/2025 → 06/15/2026 = 276 days = 0.756 yr Loss = 0.756 × 0.00375 = 0.0028 in Thickness = 0.303 − 0.0028 = 0.3002 in Wall above t-min = 0.3002 − 0.215 | 0.0852 in |
| 4 Remaining life as at 06/15/2026 | 0.0852 in ÷ 0.00375 in/yr | 22.7 yr |
| 5 Date t-min is reached | 06/15/2026 + 22.71 yr (8295 days) Check: 09/12/2025 + (0.088 ÷ 0.00375 = 23.47 yr) = 03/01/2049 | 03/01/2049 |
| 6 Compare with carried basis | 1.9 mpy: 0.088 ÷ 0.0019 = 46.3 yr from 09/12/2025 → 12/31/2071; 45.5 yr as at 06/15/2026 | Overstated by 22.8 yr |
| 7 API 653 UT interval check | Rate known: interval ≤ smaller of RCA/2N and 15 yr. RCA = 85 mils; N = 3.75 mpy → 85 ÷ 7.5 | 11.4 yr (> 5 yr plan interval) |

Years are taken as 365.25 days. Day counts are in Appendix C.

# 9. Minimum required thickness and sensitivity

## 9.1 t-min

0.215 in is the value carried in R7 (§2, §3), R8 (§4) and R9. R7 §2 attributes it to an 'API 653 §4.3 calculation in §3 of this assessment', but §3 contains no calculation. The check below uses API 653 §4.3.3.1, t-min = 2.6 (H − 1) D G / (S E):

| Input | Value | Source |
| --- | --- | --- |
| D | 55.8 ft (17 m; nameplate 56 ft) | R13 §7.1; R14 |
| H | 40 ft, full shell height. Conservative: maximum fill is 34 ft | R13 §7.1; R12 §2 |
| G | 0.79 current product (design nameplate 0.78) | R12 §1; R13 §7.1 |
| S | 23,600 psi — API 653 allowable for the bottom two courses, A283 Gr C or unknown steel (lesser of 0.80Y and 0.429T; Y = 30,000, T = 55,000 psi). The T-02-003 plate grade is not stated in R13; sister tank T-02-001 is A283 Gr C on courses 1–3. | API 653 Table 4.1; R13 §5.2, §7.2 |
| E = 1.00 | API 650 7th Ed. basic standard (radiographed per R13 §7.2). API 653 also permits E = 1.0 for corroded plate at least 1 in or 2t (whichever is greater) from welds. → t-min = 0.189 in | API 653 Table 4.2, §4.3.3.1 |
| E = 0.85 | API 650 7th Ed. Appendix A, spot RT → t-min = 0.223 in | API 653 Table 4.2 |

The record value, 0.215 in, lies between these two results. It is adopted here because the question is the rate, but the re-issued FFS must show how it was derived. Using H = 34 ft would lower t-min to 0.160 – 0.189 in.

## 9.2 Sensitivity of the date

| Case | Thickness (in) | Rate (mpy) | t-min (in) | Wall above t-min at 06/15/2026 (in) | Life at 06/15/2026 (yr) | Date t-min reached |
| --- | --- | --- | --- | --- | --- | --- |
| Adopted basis | 0.303 | 3.75 | 0.215 | 0.0852 | 22.7 | 03/01/2049 |
| Rate rounded to site one-decimal convention | 0.303 | 3.80 | 0.215 | 0.0851 | 22.4 | 11/08/2048 |
| Rate extrapolated to 6 in (linear fit, S quadrant) | 0.303 | 3.90 | 0.215 | 0.0851 | 21.8 | 04/05/2048 |
| Rate extrapolated on local 18–30 in gradient | 0.303 | 4.25 | 0.215 | 0.0848 | 20.0 | 05/27/2046 |
| Governing reading less full calibration drift | 0.297 | 3.75 | 0.215 | 0.0792 | 21.1 | 07/25/2047 |
| t-min with E = 0.85 | 0.303 | 3.75 | 0.223 | 0.0772 | 20.6 | 01/12/2047 |
| t-min with E = 1.00 | 0.303 | 3.75 | 0.189 | 0.1112 | 29.6 | 02/05/2056 |
| Combined adverse (0.297 in / 3.9 mpy / 0.223 in) | 0.297 | 3.90 | 0.223 | 0.0711 | 18.2 | 09/02/2044 |

Credible adverse combinations bring the date forward to 2044 – 2048; 03/2049 is the best-supported figure. In every case the life is well beyond the 5-year inspection interval, and no case supports the carried end-2071 date.

# 10. Consequences and recommended actions

The remaining life carried for T-02-003 (46.3 yr: R9 Summary row 11; R7 §3 'well in excess') is overstated by about a factor of two. The corrected basis is 3.75 mpy, 22.7 years as at 06/15/2026, t-min reached about 03/2049.

The shell is still above t-min (about 0.300 in against 0.215 in), and the corrected life still exceeds the planned 5-year external and UT intervals, so the rate correction raises no immediate fitness concern. However, the FFS on file does not state a defensible rate basis, and T-02-003 carries Action Required = Y for the 08/10/2026 FFS deadline (R10). R11 §2 also sets the internal interval on the 'remaining life carried in FFS', so that reference must be updated.

Finding F-01 (R8) should be closed as a datum artefact rather than carried forward as an unexplained step.

| # | Action | Purpose | Suggested owner | By |
| --- | --- | --- | --- | --- |
| 1 | Calibrated external re-survey of course 1 in service. Stations at 6, 18, 30, 48 and 72 in above the weld (= −6, 6, 18, 36 and 60 in above the ringwall) at all 8 octants, bare metal, one stated datum, each reading time-stamped against the calibration log. Confirm in the field the 12 in dimension between the weld line and the top of the ringwall (R3 §4.2) and photograph both datums; both procedures require datum photographs (R3 §6.2, R4 §5.6), and neither is on file | Validate 0.303 in; find the 8-octant minimum at 6 in; first like-for-like rate after the event (about 13 months after 06/16/2025); start a rate history at 6 in | MI Engineer (J. Merchant) | Before 08/10/2026 |
| 2 | Query Ashgrove on: re-shoot status of readings voided at 09/12 17:45; calibration record for s/n AG-1147; D. Ferris's certification; identity of reviewer 'M. Okafor'; T-02-003 datum photograph and job instruction (R3 §4.2, §6.2); annotated datum transposition table (R3 §4.4) | Close the Section 6 issues | MI Engineer / Project Engineer | 07/2026 |
| 3 | Retrieve the original CB-UT-2021-019 report and confirm its grid and datum | Confirm the basis of the 3.75 mpy rate | Eng. Administrator / MI Engineer | 07/2026 |
| 4 | Re-issue FFS TC-MECH-FFS-0059 stating: the rate basis and interval; the governing location with its datum; the t-min derivation; the current wind basis (70 m/s, TC-CIV-WIND-0045, replacing the superseded 45 m/s). Reconcile with tracker entry TC-MECH-FFS-0121 | Meet TC-MECH-PROC-0007 §4 | MI Engineer; certify S. David | Before 08/10/2026 |
| 5 | Correct workbook TC-MECH-CALC-0075 (T-02-003 sheet; Summary row 11) and add a datum column to every sheet | Remove 1.9 mpy / 46.3 yr | MI Engineer; check J. Peters | With action 4 |
| 6 | Re-issue inspection report TC-MECH-INSP-0163 under a current certificate. It was signed 01/14/2026, during the API 653 lapse of 11/30/2025 – 02/20/2026 (R19; R18 §3). Correct F-01 and the UT method description | Certified evidence | S. David | Before 08/10/2026 |
| 7 | Fill in the blank 'Grid density and datum convention' bullets in R11: one site datum (or mandatory transposition), and the owner's grid to be re-read by any contractor | Prevent recurrence | MI Engineer | Plan Rev B |
| 8 | Complete the category reassessment for PCN-2025-039 (R18 §5.2). Resolve the conflicting vapour-space descriptions: PCN 'N2 blanket 15 mbar', PDS 'not fitted', MOC-2023-031 VRU tie-in | Basis for corrosivity after the event | Process Engineer / MI Engineer | 07/2026 |

# 11. Other record observations (do not change the result)

R7 §2 cites the construction pack's 'fabricator MTR set' for the 0.375 in thickness, but the pack does not contain MTRs (R13 §1). The 0.375 in nominal is supported by R14, which notes it is a design-drawing value, not a measured one.

R7 §4 uses 45 m/s from the 2023 wind study, superseded from 01/2026 by 70 m/s (TC-CIV-WIND-0045). The FFS tracker (R20) lists 'TC-MECH-FFS-0121, signed 01/20/2026, 70 m/s' for T-02-003. That is not the FFS on file, and the date falls within the certificate lapse.

R8 §3 describes a Panametrics V112-RM single-element 0.25 in transducer with averaged readings, whereas R1 and R3 record an Olympus D790 dual-element probe and the minimum of three readings. R8's inspector (K. Berensen) is not on R1's personnel list.

R8 F-05 says the process data sheet does not reflect PCN-2025-039, but R12 (Rev 2, 06/24/2025) does.

The Ashgrove calibration log ends 09/15/2025 (R5), yet R9 records Ashgrove readings on 09/16 (T-03-002) and 09/17 (T-03-003), and R6 is dated 09/17.

Course 5 readings on 09/12/2025 were taken from the ladder cage without proper access, and the site fitter declined to accept them (R20, fitter note '12/9'). Course 5 does not govern.

Last internal inspection: R11 gives 2018 (Cordell Bayou legacy), while R10 gives 11/02/2019.

R9 gives t-min of 0.171 / 0.127 in for courses 2 / 3, against 0.170 / 0.140 in R8.

# Appendix A — Course 1 readings on a common datum, all quadrants

| Quad. | Elev. above weld (in) | MF grid | AG grid | CB 04/2021 (in) | MF 04/28/2025 (in) | AG 09/12/2025 (in) | Loss 21→25 (in) | Rate 21→25 (mpy) | Change MF→AG (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N | 6 | — | C1-N-6 | — | — | 0.306 | — | — | — |
| N | 18 | C1-N-6 | — | 0.352 | 0.337 | — | 0.015 | 3.75 | — |
| N | 24 | — | C1-N-24 | — | — | 0.343 | — | — | — |
| N | 30 | C1-N-18 | — | 0.363 | 0.350 | — | 0.013 | 3.25 | — |
| N | 48 | C1-N-36 | C1-N-48 | 0.372 | 0.361 | 0.359 | 0.011 | 2.75 | 0.002 |
| N | 72 | C1-N-60 | C1-N-72 | 0.380 | 0.371 | 0.369 | 0.009 | 2.25 | 0.002 |
| E | 6 | — | C1-E-6 | — | — | 0.304 | — | — | — |
| E | 18 | C1-E-6 | — | 0.350 | 0.335 | — | 0.015 | 3.75 | — |
| E | 24 | — | C1-E-24 | — | — | 0.341 | — | — | — |
| E | 30 | C1-E-18 | — | 0.361 | 0.348 | — | 0.013 | 3.25 | — |
| E | 48 | C1-E-36 | C1-E-48 | 0.370 | 0.359 | 0.357 | 0.011 | 2.75 | 0.002 |
| E | 72 | C1-E-60 | C1-E-72 | 0.378 | 0.369 | 0.367 | 0.009 | 2.25 | 0.002 |
| S | 6 | — | C1-S-6 | — | — | 0.303 | — | — | — |
| S | 18 | C1-S-6 | — | 0.349 | 0.334 | — | 0.015 | 3.75 | — |
| S | 24 | — | C1-S-24 | — | — | 0.340 | — | — | — |
| S | 30 | C1-S-18 | — | 0.360 | 0.347 | — | 0.013 | 3.25 | — |
| S | 48 | C1-S-36 | C1-S-48 | 0.369 | 0.357 | 0.356 | 0.012 | 3.00 | 0.001 |
| S | 72 | C1-S-60 | C1-S-72 | 0.377 | 0.367 | 0.366 | 0.010 | 2.50 | 0.001 |
| W | 6 | — | C1-W-6 | — | — | 0.305 | — | — | — |
| W | 18 | C1-W-6 | — | 0.351 | 0.336 | — | 0.015 | 3.75 | — |
| W | 24 | — | C1-W-24 | — | — | 0.342 | — | — | — |
| W | 30 | C1-W-18 | — | 0.362 | 0.349 | — | 0.013 | 3.25 | — |
| W | 48 | C1-W-36 | C1-W-48 | 0.371 | 0.360 | 0.358 | 0.011 | 2.75 | 0.002 |
| W | 72 | C1-W-60 | C1-W-72 | 0.379 | 0.370 | 0.368 | 0.009 | 2.25 | 0.002 |

MF = Merrowfield; AG = Ashgrove; CB = Cordell Bayou record CB-UT-2021-019 (Merrowfield baseline column). Sources: R1 sheet T-02-003 rows 10–29; R2 sheet T-02-003 rows 10–25. Ashgrove's NE, SE, SW and NW octants were read at 24 in only: 0.341, 0.342, 0.344, 0.343 in.

# Appendix B — Courses 2–5

| Course | MF grid (elev. above weld) | CB 04/2021 (in) | MF 04/2025 (in) | Loss (in) | Rate (mpy) | AG grid (elev. above weld) | AG 09/2025 (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | C2-x-19 / C2-x-58 (127 / 166 in) | 0.304 – 0.306 | 0.295 – 0.297 | 0.009 | 2.25 | C2-x-24 (120 in) | 0.293 – 0.295 |
| 3 | C3-x-38 (242 in) | 0.246 – 0.247 | 0.239 – 0.241 | 0.006 – 0.007 | 1.50 – 1.75 | C3-x-24 (216 in) | 0.237 – 0.239 |
| 4 | C4-x-38 (338 in) | 0.247 – 0.248 | 0.243 – 0.245 | 0.003 – 0.004 | 0.75 – 1.00 | C4-x-24 (312 in) | 0.241 – 0.243 |
| 5 | C5-x-38 (434 in) | 0.248 – 0.249 | 0.245 – 0.247 | 0.002 – 0.003 | 0.50 – 0.75 | C5-x-24 (408 in) | 0.243 – 0.245 |

Abbreviations as Appendix A. No Merrowfield and Ashgrove stations coincide on courses 2–5. Ranges cover the N, E, S and W quadrants. Sources: R1, R2 sheets T-02-003.

# Appendix C — Date arithmetic

| Interval | Days | Years (÷ 365.25) | Used in |
| --- | --- | --- | --- |
| 09/14/1987 (in service, R13 §7.3) → 09/12/2025 | 13878 | 37.996 | Basis A: (0.375 − 0.303) ÷ 38.0 = 1.9 mpy |
| 09/14/1987 → 04/28/2025 | 13741 | 37.621 | Basis B: (0.375 − 0.334) ÷ 37.6 = 1.1 mpy |
| 04/2021 → 04/28/2025 | — | 4.0 (month to month; 2021 day not recorded) | Basis F: 0.015 ÷ 4.0 = 3.75 mpy |
| 04/29/2025 → 09/12/2025 | 136 | 0.372 | Bases C and E |
| 06/16/2025 (event) → 09/12/2025 | 88 | 0.241 | Post-event part of basis E |
| 09/12/2025 → 06/15/2026 | 276 | 0.756 | Loss to the assessment date |
| 06/15/2026 → 03/01/2049 | 8295 | 22.71 | Remaining life at the assessment date |

# Review and sign-off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared (engineering review) |  |  | 06/15/2026 |
| Checked | James Merchant, Mechanical Integrity Engineer | pending |  |
| Reviewed (API 653) | Steve David, API 653 Cert. No. 653-38472, expiry 02/20/2029 | pending |  |

This review relies on the records listed in Section 2 as held on 06/15/2026. It does not replace the certified FFS. Its conclusions should be carried into the re-issue of TC-MECH-FFS-0059 and workbook TC-MECH-CALC-0075.